Standard operating procedure for  CRYOGENS AND COLD BATHS

File: SOP-cryogens.doc

Last changed: 03-Jan-15 (aph)

NEED for SOP:

Liquid N2  in ~1 L dewars (and smaller volumes) is occasionally used in ABB-332 and there is a 60 L liquid N2 in the lab, which may be filled if there is a need for frequent use (it has not been used for ~8 years though)

Liquid N2 can give skin burns if it spills onto a freezes your clothes in such a way you cannot remove them. There is also a remote hazard of asphyxiation if all of the liquid N2 in our dewar were to expand into the room at very short time interval.

Safe handling of liquid N2:

Always were safety glasses when working with Liquid N2.

It is safer to allow liquid N2 to spill on your bare skin than on any restrictive piece of clothing such as gloves or a lab coat. Thus, unless you are totally paranoid, don’t get all suited up to handle small amounts of liquid N2.  On the other hand, I would not take a bath in liquid N2.

Filling large volume dewar

This is done by stores personnel in the loading dock. If you need to get the dewar filled, take it down to the loading dock and notify the stores person. When it is filled, you can bring it up to ABB-332. Be sure to wear a lab coat and eyeglasses when bringing the dewar to ABB-332 from the lABB loading dock.