Standard operating procedure for  Heat Sources

File: SOP-heat_sources.doc

Last changed: 03-Jan-15 (aph)

NEED for SOP:

Heat sources (> 100C) that exist in ABB-333 include

- Diffusion pump heaters

- Oxy-acetylene torch

- Muffle furnace

- “Benzomatic” propane torch

- mini-solder torch

- Soldering iron (also in ABB-332 in electronics area)

Hazards

- Skin burns if come into contact with the heat source.

- Exposure to volatile Pb from Pb-Sn solder

Precautions

- Be aware when anyone in the lab is using one of the above hot units. Stand back.

- If you are the one using the heat source, follow proper operating procedures for the item.

- Always turn O2 on last and turn it off first when using O2 - gas, or O2 - acetylene torch. The reverse order can easily lead to an explosive gas mixture in the torch area.

- Always use tinted glasses when using the O2 - acetylene torch.

- The diffusion pump heaters on ISEELS and Surface-EELS (when on) are at a temperature of ~250 C, similar to that of the element on a stove. You will feel its heat as your skin approaches the element. Do not touch.

In emergency situations such as fires, gaseous emissions or spills:

- Notify emergency workers (Dial 88 on phone in ABB_333)

- If you were using the oxy-acetylene torch, turn off  the main regulator of the Oxygen and Acetylene cylinders and the natural gas spigot prior to leaving the room.